| SECTION H CLADDING / COVERING | Product and manufacturer’s details | Ability to be disassembled, reused, recycled, downcycled YY=yes, easily YH= yes, with high work input N=no U=unknown V-varies according to product | Relevant system features | Non-disassembleable / reusable / recyclable alternatives | Issues to consider | Advantages of reusable/ recyclable option | Frequency of use / examples of reuse / recycling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CURTAIN WALLING | Main use: external walls Product: metal box section systems Manufacturer: varies | Reuse as new – YH Reuse 2nd hand – YY Recycling – YH Downcycle – YH | Infill panels are fixed within the metal box sections with rubber gaskets | Mastic bonded glass can not be reused and the separation of the building elements to enable recycling is time consuming and difficult | The gaskets should be easily removable in order to facilitate the recycling of the metal sections. When reusing elements the dimensions of the elements to be reused will have to be considered when designing. | Maintenance is greatly facilitated as well as the potential for upgrading the glass elements with improvements in technologies. | Common use |
| STRUCTURAL GLASS ASSEMBLIES | Main use: wall / roofs Product: varies | Reuse as new – YH Reuse 2nd hand – YH Recycling – YH Downcycle – YH | Glass elements are connected with spider connectors. The joints between the glass elements are sealed with mastic | Systems with extensive use of mastic are more difficult and time consuming to disassemble. | When reusing elements the dimensions of the elements to be reused will have to be considered when designing. | Maintenance is facilitated | Common use |
| TIMBER WEATHERBOARDING | Main use: external walls Product: screw fixed boarding | Reuse as new – YH Reuse 2nd hand – YY Recycling – N Downcycle – YY | The timber boarding is fixed with screws to the substructure | Nailed and secrete nailed boards are difficult to remove intact and reused | The choice of fixing material will affect the durability of the installation. Timber can not be recycled (implying into an equivalent product), but can be downcycled and made into a board material e.g. chipboard Timber also has a calorific value and can be burnt and it also decomposes over time. However in both these case care should be taken if the timber has been treated, as some treatment material can leach out into nature and are toxic. | Maintenance is facilitated | Common use |
| RIGID SHEET CLADDING (PLY, PLASTICS ETC) | Main use :external walls Product: cladding fixed with exposed fasteners Manufacturer: SFS Stadler www.sfs-online.com/construction | Reuse as new – YH Reuse 2nd hand – YY Recycling – V Downcycle – V | Cladding with exposed fixings | Cladding bonded or fixed with hidden self-tapers or nails can not be dismantled easily without damage. Reuse is therefore difficult. Recycling may be possible depending on the material used | Visible fixings may be liable to corrosion and tampering. Fasteners with integrated covers will protect fixing and may be aesthetically more acceptable. | Maintenance and refurbishment work is facilitated. Installation is quick compared to wet finishes e.g. render | Common use |
| CLAY CLADDING | Main use :external walls Product: clay tiling Manufacturer: various | Reuse as new – YH Reuse 2nd hand – YY Recycling – N Downcycle – YY | Clay tiling elements fixed on steel or timber support system | Cladding bonded to substrate can not be reused or recycled |  | Maintenance and refurbishment work is facilitated. | Common use |
| METAL PROFILES/ FLAT SHEET CLADDING/ COVERING | Product: metal cladding panel system Main use: commercial external walls Manufacturer: Euroclad www.euroclad.com | Reuse as new – YH Reuse 2nd hand – YY Recycling – YH Downcycle – NA | Metal composite panel with rockwool core. Fixings rely on panels overlapping over each other with minimal fixings. | Composite panels with PIR/PUR are more difficult to recycle, the separation of plastic core from the metal external layer may result in contaminants in the metal | Rockwool and steel and aluminium can be recycled. Steel’s magnetic qualities make the separation of steel from other materials efficient and cost-effective. Coated steel can be recycled by using the basic oxygen steel-making process. Steel coatings are now available in low VOC water based formulations. The rockwool core can be recycled for lower grade applications. The panels can also be reused, however the sizes of the panels will have to be taken into account when designing the new building. | The relatively large panels and the quick fixing method makes the installation and dismantling a quick process. | Common use |
| NATURAL STONE SLAB CLADDING/ FEATURES | Main use: external walls Product: cladding fixed with metal cramps | Reuse as new – YH Reuse 2nd hand – YH Recycling – N Downcycle – YY | Cladding fixed with metal cramp and open joints | Cladding fixed with adhesive |  |  |  |
| PLAIN ROOF TILING ETC. | Main use: roofing Product: tiling on battens | Reuse as new – V Reuse 2nd hand – YY Recycling – YY Downcycle – YY | Tiles of various materials fixed with metal clips and or nails using dry ridge and eves details | Roof eves bedded in mortar can prove more difficult to dismantle than mechanically fixed tiles | Reusing and recycling roof tiles is very common | The use of dry details eliminates risk of mortar failure, mechanical fixings ensure reliable fixing against uplift. It may prove simpler to build and eliminates wet trades reducing costs, ridge line can incorporate ventilation This installation methods also facilitates replacement and repair, reduces maintenance UK produced tiles (clay and slate etc) make use of local labour through manufacturing process and mining. | Use common Reuse common Recycling common |
|  | Main use: roofing Product: metal tile look-alike panels Manufacturer: Decra www.decra.co.uk | Reuse as new – YH Reuse 2nd hand – YY Recycling – YY Downcycle – NA | DEKRA metal clay tile look-alike panels fixed mechanically | Traditional tiling as above and with eves bedded in mortar. | Removing metal panels at roof level is often associated with damage to the panels and therefore recycling is a more realistic end of use solution. Panels can be laid at 10/15° | Larger elements with fewer fixings increase the speed of construction and amount of work at heights reducing risk to health and safety as well as cost. Panels are one seventh of the weight of concrete and therefore require reduced less costly roof structure and are well suited to refurbishments where additional floors are added to an existing building. | Use increasing |
| METAL SHEET COVERINGS/ FLASHINGS | Main use: roof covering Product: mechanically fixed roofing sheets | Reuse as new – YH Reuse 2nd hand – YY Recycling – YY Downcycle – NA | Metal sheet covering are fixed mechanically independently from insulation | Composite roofing panels are more work-intensive to recycle Tiled roofs are more time consuming to dismantle | Metal has a high waste value and is generally already recycled | Waste roofing material has a resale value. | Common use Common recycling Reuse not viable |