| SECTION J WATER-PROOFING | Product and manufacturer’s details | Ability to be disassembled, reused, recycled, downcycled YY=yes, easily YH= yes, with high work input N=no U=unknown V-varies according to product | Relevant system features | Non-disassembleable / reusable / recyclable alternatives | Issues to consider | Advantages of reusable/ recyclable option | Frequency of use / examples of reuse / recycling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SINGLE LAYER POLYMERIC ROOF COVERING | Main use: flat roofing Product: single ply mechanically fixed membrane | Reuse as new – YY Reuse 2nd hand – YY Recycling – YY Downcycle – YY | Single ply membrane fixed mechanically | Single ply membrane fixed with adhesive, liquid applied finish, | Reused membranes would not have a guarantee. Mechanically fixed roofing membranes can be easily dismantled and potentially reused, however the reuse is limited to a similar design as the one dismantled or alternatively the membrane will have to be reworked to fit a new design. Recycling of membranes depends on the material of the membrane | Mechanical fixing avoids use of adhesives and the contact with volatile organic compounds. | Limited use Reuse unusual Recycling unusual |
|  | Main use: roofs, canopies Product: proprietary Manufacturer: Fabric architecture www.fabricarchitecture.com Manufacturer: Tensarc www.tensarc.co.uk | Reuse as new – N Reuse 2nd hand – YY Recycling – YY Downcycle – YY | Tensile membrane fixed at perimeter and as necessary with mechanical fixings |  | Issues as for single ply membrane Aesthetics will affect the possibility of reuse of the membrane. ETFE membranes installed in smaller unit sections may prove more flexible in terms of potential for reuse on a different project. | Maintenance, such as replacement of damaged sections is facilitated The replacement of the complete membrane can be undertaken quickly. | Use increasing ETFE installation at Kingsdale School designed to be reused |