| SECTION K LINING, SHEATHING, DRY PARTITIONING | Product and manufacturer’s details | Ability to be disassembled, reused, recycled, downcycled YY=yes, easily YH= yes, with high work input N=no U=unknown V-varies according to product | Relevant system features | Non-disassembleable / reusable / recyclable alternatives | Issues to consider | Advantages of reusable/ recyclable option | Frequency of use / examples of reuse / recycling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RIGID SHEET FLOORING /SHEATHING/ SARKING /LININGS/ CASINGS | Main use: floors Product: screw fixed sheathing | Reuse as new – YY Reuse 2nd hand – YY Recycling – YY Downcycle – YY | Standard sized panels fixed with mechanical fixings without adhesive to receive finish | Adhesive fixed |  | Maintenance is facilitated through providing easy access to under floor services etc. | Common use Common reuse Recycling limited |
| RIGID SHEET TIMBER/ WOOD & PLASTICS VENEERED PANELLING & LINING | Product: Neatmatch Main use: wall lining Manufacturer: Neat concepts www.neatconcepts.co.uk | Reuse as new – YH Reuse 2nd hand – YY Recycling – YY Downcycle – YY | MDF profiled boarding to line walls, edges are profiled. | Timber boarding fixed with adhesives. Plastered walls. Plasterboard walls screwed and covered | Aesthetics are distinct and not acceptable to all. | Installation is dry and a quick process. Access to services behind wall can be facilitated. | Limited |
|  | Product: Parklex Main use: wall lining Manufacturer: Composites Gurea | Reuse as new – YH Reuse 2nd hand – YY Recycling – U Downcycle – YY | Rigid board finish fixed with secret mechanical fixings | Timber panelling bonded to substrate would preclude the reuse and often the recycling of timber boards as their removal would involve their damage. | Some damage to the panels can occur when removed, therefore careful manhandling is essential. Using the largest size of boards possible will maintain the maximum flexibility when the boards come to be reused. Also the larger the units to dismantle the quicker the process and the most cost-effective the reuse. | The ability to easily reposition wall panels facilitates the re-planning of spaces and the access to services for maintenance. | Commonly used |
|  | Product: retail display wall panels Main use: retail Manufacturer: various | Reuse as new – YY Reuse 2nd hand – YY Recycling – V Downcycle – V | Display wall system consisting of a frame, panels and fixing systems | Display systems making use of the structural wall as part of their display | The ceiling is unaffected by the display wall and has to be considered. | In retail areas the displays are changed far in advance of the display systems becoming unusable, reuse of these systems is therefore financially beneficial. The ability to easily reposition wall panels facilitates the re-configuration of retail displays, spaces and service. | Commonly used |
|  | Product: Marbex wall panels Main use: lining of bathroom, kitchen and other walls and ceilings Manufacturer: Swish Building Products | Reuse as new – YH Reuse 2nd hand – YY Recycling – U Downcycle – U | Rigid plastic veneered boards fixed with mechanical fixings and visible jointing strips | Ceramic tiling is not reusable or recyclable and is difficult to downcycle due to the grout contamination of the tiles, which can otherwise be considered inert masonry waste and can be used | Wall boards are room height and ceiling boards are 4m long, but the width is only 250mm therefore numerous joints will be visible Aesthetics of the boards are not what is often expected in rooms other than bathrooms Marbex achieve Class 1Y fire rating and conforms to the requirements of the Food Safety Act 1990 and General Hygiene Regulations 1995 | Speed of installation – requires half the time of tiling Allows access to wall for maintenance | Commonly used |
| RELOCATABLE PARTITIONS | Product: office partitions Main use: commercial Manufacturer: varies | Reuse as new – YH Reuse 2nd hand – YY Recycling – YH Downcycle – V | System generally is made of a framing element and infill panels, including solid and glazed panels, generally fixed to the framing systems mechanically | Traditional partitions can be designed to be reusable, but in the main are not. | Most relocatable partitions are reusable and depending on the materials used recyclable Framing should be of one material (eg steel or aluminium not mixed) to enable the recycling of the system The elements to be reused have to be sturdy enough to not suffer in terms of aesthetics and performance when dismantled. The fixings have to be designed to be easily unfixed and refixed and not wear out (note screw threads) | Provides flexibility in commercial office and retail and other commercial spaces. Provides opportunities for customising spaces with different infill panels and upgrading spaces without having to change all fixtures. | Common use Common reuse Recycling unusual |
| MOVABLE PARTITIONS | Product: Movable partitions Main use: all types of buildings, but mainly to split large meeting rooms into move than one small room | Reuse as new – YH Reuse 2nd hand – YY Recycling – V Downcycle – V | The wall panels are fixed with door ironmongery and are designed to slide or fold out of the way. The screw fixings can easily be dismantled, however this is time-consuming | Movable partitions are by their nature likely to be easily dismantled, making it easy to reuse the wall panels, however whether these can be recycled will depend on whether they are composite boards or mainly made of one material | Acoustical movable partitions are available to provide good acoustic separation if required. | Movable partitions provide a flexible use of space, which can prove economically beneficial as a number of room requirements can be accommodated in one flexible space. | Common use |
| PLASTERBOARD FIXED PARTITIONS/ INNER WALLS / LININGS | Product: Multipanel Main use: internal walls Manufacturer: Grant Westfield www.grantwestfield.co.uk | Reuse as new – YH Reuse 2nd hand – YY Recycling – N Downcycle – N | Wall linings systems made with tongue and groove or square edge panels screwed on timber backing framework. | Plasterboard partitions (timber and metal) are very difficult to reclaim and recycle. Plasterboard is virtually impossible to reuse, even if screw fixed, once the edges are taped and jointed the screws become inaccessible and the time required to release a panel makes the reuse financially unfeasible. | Multipanels are composite panels therefore not recyclable, however the design of the system could be used to develop recyclable panels Aesthetically this product is unusual and could prove unacceptable to some. | Provides flexibility for internal layouts. Facilitates maintenance and access to services in walls. | limited use |
| GLASS BLOCK WALLING | Main use: internal walls Product: polystar Manufacturer: Rockwell Sheet Sales www.rockwellsheet.com | Reuse as new – YY Reuse 2nd hand – YY Recycling – YY Downcycle – YY | Interlocking polycarbonate blocks with clip fixings | Glass blocks set in mortar are liable to break when dismantled. | Aesthetics, durability and u-values have to be assessed for each particular use. | The system is very light therefore easy to handle and improving working conditions. Also mortar is not required and its abrasive characteristics avoided. Provides flexible interior layouts. | Limited use |
| FRAMED PANEL CUBICLE PARTITIONS | Main use: toilets and changing rooms Product: single material (e.g. steel, ply) partitions Manufacturer: various | Reuse as new – N Reuse 2nd hand – YY Recycling – V Downcycle – V | Cubicle partitions consisting of boards and mechanical connectors and / or frames | Many cubicle partitions are dismantleable and potentially reusable, but most are made with laminated composite boards which can not be recycled | Fixing types can affect the reusability of the system Metal blade slots into timber panel – this detail is likely to damage panel when removed Fixing elements (EG ironmongery) should not need to be unscrewed from the board when dismantled, but rather unclipped from a second counterpart fixing element. This avoids screw holes in the boards from becoming enlarged and unusable. | Flexibility of layout Replacement of damaged boards is made easy Metal partitions have waste value when no longer used | Limited use |
| SUSPENDED CEILINGS | Main use: commercial Product: metal suspended ceilings Manufacturer: various | Reuse as new – YH Reuse 2nd hand – YY Recycling – YY Downcycle – NA | Metal ceiling tiles set within a metal suspension system | Composite (e.g. metal and acoustic material) tiles within suspension system | Reuse is very simple, but colour variations might occur over time. Metal tiles may be damaged (mainly dented and bent) while being dismantled | Suspended ceiling systems allow for internal layout flexibility as well as reconfiguration of services Facilitates the commissioning of services and their replacement and maintenance | Common use Reuse limited |
|  | Main use: commercial Product: acoustic suspended ceilings Manufacturer: various | Reuse as new – YH Reuse 2nd hand – YY Recycling – N Downcycle – N | Acoustic ceiling tiles loosely set within a metal suspension system | Composite (e.g. metal and acoustic material) tiles within suspension system | Acoustic tiles need to be resurfaced if they are to be reused in a commercial environment, and there are companies that undertake tile refurbishments. Acoustic tiles are fragile and easily damaged | As above | Common use Reuse limited |
| RAISED ACCESS FLOORS | Main use: commercial Product: solid HDF boards Manufacturer: various | Reuse as new – YY Reuse 2nd hand – YY Recycling – YY Downcycle – YY | HDF boards screw fixed or loose laid on metal pedestals | Composite boards made with plastic or metal facing on HDF or MDF can be reused, but not recycled | Compared to faced boards solid HDF may become worn more quickly | Enables provide a flexible layout Facilitates the commissioning of services and their replacement and maintenance | Common use Reuse limited |