| SECTION L WINDOW, DOORS, STAIRS | Product and manufacturer’s details | Ability to be disassembled, reused, recycled, downcycled YY=yes, easily YH= yes, with high work input N=no U=unknown V-varies according to product | Relevant system features | Non-disassembleable / reusable / recyclable alternatives | Issues to consider | Advantages of reusable/ recyclable option | Frequency of use / examples of reuse / recycling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WINDOWS/ ROOFLIGHTS/ SCREENS/ LOUVRES | Main use: general Product: solid material windows Manufacturer: various | Reuse as new – YH Reuse 2nd hand – YY Recycling – V Downcycle – V | Solid material e.g. timber, aluminium or steel windows screw fixed and sealed with compressible fillers | Timber windows with applied finishes such as PVC coatings or composite materials such as timber and steel make the recycling and repair of the windows difficult. | Windows in general have the potential to be reused as a building element and this is made easier by avoiding mastic sealants and installing compressible fillers. This may however have an effect on the air tightness of the building. Recycling of the windows depends on having a solid material compound (e.g. timber and aluminium or polyester coating ) are difficult to recycle. | Solid timber windows can be easily repaired if damaged. | Common use |
| DOORS/ SHUTTERS/ HATCHES | Main use: general Product: solid material doors Manufacturer: various | Reuse as new – YH Reuse 2nd hand – YY Recycling – V Downcycle – V | Door leaf is a solid material (e.g. timber, steel) and fixed mechanically to frame | Composite steel/ insulation/ timber/ honeycomb cardboard doors can not be recycled and are more difficult to refurbish for reuse | All doors tend to be easily dismantled and reuse is possible. Materials used for doors are generally metal, which is very recyclable and timber, which can be easily downcycled. | Solid doors can be refurbished thus have a potential for a long life. | Common use Common reuse |