| SECTION N FURNITURE EQUIPMENT | Product and manufacturer’s details | Ability to be disassembled, reused, recycled, downcycled YY=yes, easily YH= yes, with high work input N=no U=unknown V-varies according to product | Relevant system features | Non-disassembleable / reusable / recyclable alternatives | Issues to consider | Advantages of reusable/ recyclable option | Frequency of use / examples of reuse / recycling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GENERAL FIXTURES/ FURNISHINGS/ EQUIPMENT | Product: kitchen cabinets Main use: kitchen | Reuse as new – YH Reuse 2nd hand – YY Recycling – V Downcycle – V | Free-standing kitchen cabinets | Fully fitted kitchen cabinets. Some fitted units can be disassembled and reused or recycled, however the disassembly process can damage the units and the units may not fit in a new location. | Free-standing units will facilitate the reuse of units, however the recycling of the units will depend on the details of the materials used. E.g. composite boards are impossible to recycle efficiently. | Enables a more flexible layout. | Use increasing |
|  | Product: Microslat - Retail display systems Main use: retail Manufacturer: Proloc www.proloc-online.com | Reuse as new – YH Reuse 2nd hand – YY Recycling – YY Downcycle – YY | Interlocking wall panels 125mm high and up to 6000mm long. | Systems fixed to the wall with individual supports. | The aesthetics of the system have to be considered and are appropriate for retail, perhaps less for offices. The grooved slats will collect dirt. | The speed of installation is high. The system can be used to cover the whole wall surface making the substrate quality not crucial. | Common |