| SECTION P BUILDING FABRIC SUNDRIES | Product and manufacturer’s details | Ability to be disassembled, reused, recycled, downcycled YY=yes, easily YH= yes, with high work input N=no U=unknown V-varies according to product | Relevant system features | Non-disassembleable / reusable / recyclable alternatives | Issues to consider | Advantages of reusable/ recyclable option | Frequency of use / examples of reuse / recycling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SUNDRY INSULATION/ PROOFING WORK/ FIRE STOPS | Main use: thermal and acoustic insulation Product: wall, roof and floor insulation / rigid and non-rigid | Reuse as new – V Reuse 2nd hand – V Recycling – V Downcycle – V | Loose laid insulation | Insulation that has been bonded to another material can be difficult to separate from the other material and therefore not recyclable. However increasingly processes are being developed to separate composite materials. | Loose insulation made of expanded and extruded polystyrene, rockwool, cellulose fibre, blown and spun glass can be recycled. Loose insulation made with polyurethanes, polyisocyanurates and phenolic foams are difficult to recycle. | Where the insulation is loose laid, for example on a roof it is possible to replace the insulation when damaged or when an upgrade is necessary. | Common |
| UNFRAMED ISOLATED TRIMS/ SKIRTINGS/ SUNDRY ITEMS | Product: skirtings | Reuse as new – V Reuse 2nd hand – YY Recycling – V Downcycle – V | Timber, metal and plastic skirtings fixed with screws to substrate | Skirtings fixed with adhesive to substrate | Removing skirtings fixed with adhesive will damage both substrate and skirting. Removing skirtings fixed with screws is likely to cause less damage and if screws are left exposed can cause no damage at all. | Facilitates changes to layouts and decorations. Avoids emissions from adhesives during installation process. | Common Exposed screws are seldom used |
| HOLES/ CHASES/ COVERS/ SUPPORTS FOR SERVICES | Product: Heat Profile Main use: domestic Manufacturer: Heat Profile www.heatprofile.co.uk | Reuse as new – YH Reuse 2nd hand – YY Recycling – U Downcycle – U | Profiled skirting element fixed on brackets and integrating heating elements and other services | Services installed in walls or floors Skirtings fixed with adhesive to wall | In order to integrate service elements the skirting profile is slightly larger than a normal skirting | Allows flexibility of services including IT, electrical and heating within a space. Removes the need for radiators thus allowing furniture to be freely positioned Refurbishment work is facilitated through easy access to services and easy dismantleability to move walls. | Limited |
|  | Product: cable management system Main use: commercial Manufacturer: various | Reuse as new – YH Reuse 2nd hand – YY Recycling – U Downcycle – U | Metal or plastic profiled trunking, installed at dado or skirting level, fully accessible and integrating sockets etc | Services installed in walls or floors | Aesthetics has to be considered as the trunking may appear utilitarian Laminated profiles can be difficult to recycle and even downcycle | Allows flexibility of services including IT, electrical and heating within a space. | Common use Common reuse |
|  | Product: plug in lighting system Main use: commercial Manufacturer: various www.flexconnectors.co.uk www.hager.co.uk | Reuse as new – YY Reuse 2nd hand – YY Recycling – YH Downcycle – N | System consists of pre-wired elements that can be plugged in a distribution element | Hard wired services |  | Installation time is reduced Allows flexibility of services Installations can be easily upgraded | Increasing use |